Year 12 ATAR PHYSICS

Practical Test 	WAVES AND QUANTA

Part B: DETERMINING THE EFFECTIVE WAVELENGTH OF A LED.

Marks:	     /10

ANSWERS

The threshold voltage at which a semiconductor LED turns on gives a measure of the bandgap or emission wavelength of the LED through the following relationship:

The following circuit was constructed:

V

V

100 Ω resistor

50K Rheostat

100 Ω resistor

50K Rheostat

A

A

By adjusting the rheostat, the following values of current and voltage through the LED was recorded:

| V(V) | I(mA) |
| --- | --- |
| 2.40 | 0.12 |
| 2.43 | 0.40 |
| 2.44 | 0.50 |
| 2.48 | 0.97 |
| 2.50 | 1.20 |

Question 1

Plot a suitable graph of the results. 							( 4 marks)

mA

1.2

1.1

1.0

0.9

0.8

0.7

0.6

0.5

0.4

0.3

0.2

0.1

0

mA

1.2

1.1

1.0

0.9

0.8

0.7

0.6

0.5

0.4

0.3

0.2

0.1

0

Labels   (1) appropriate scales  (2) accurate plotting     (-1 if scale inappropriate and LOBF too steep)

x

x

x

x

x

x

x

x

x

x

Range: 2.39 - 2.42

2.40                                   2.45                                       2.50

2.395

2.40                                   2.45                                       2.50

2.395

Question 2                                                                                          				( 3 marks)

Determine the threshold voltage from the graph. Clearly show your method on the graph.

(1)   Method shown

Vth = 2.39V to 2.42V				(1)   (0 above or below)

Answer to 3 or 4 sig figs			(1)

Question 3                                                                                                       			( 3 marks)

Determine the wavelength of the LED in nanometres.

= hc / eVth

= 6.63 x 10-34 . 3 x 108		(1)

1.6 x 10 -19 . 2.395

=  5.19 x 10-7 m

=  5.19 x 102 nm   (3 sig figs)

- (1)

Note for example: 519nm 3 sig fig OK

520 nm only 2 sig fig    -1